ROSE5720 Experimental Robotics

Final Project Report Miniduck Robot Project

Name:  WANG Zihan

Student ID: 1155249290

Abstract

This project demonstrates the full hardware-to-deployment pipeline for the Open Duck Mini v2, a miniature bipedal robot modelled after the Disney BDX droid. The robot was 3D-printed from 37 STL parts, assembled with 14 Feetech STS3215 servo motors, and calibrated. A locomotion policy was trained in MuJoCo MJX using PPO with an imitation reward over 150-300 million environment steps. The trained ONNX policy was deployed on a Raspberry Pi Zero 2W embedded controller at 50 Hz. The robot achieved stable forward walking on flat terrain responding to joystick velocity commands. Key challenges — motor backlash, communication latency, and IMU orientation — were addressed through domain randomisation, backlash joint modelling, and per-robot calibration workflows.

1  Introduction

This project replicates, assembles, trains, and deploys the Open Duck Mini v2 — a miniature bipedal walking robot modelled after the Disney BDX droid. The robot stands approximately 42 cm tall with its legs extended and is built from 3D-printed structural parts, 14 Feetech STS3215 serial-bus servo motors, and a Raspberry Pi Zero 2W as the onboard computer, with a total Bill-of-Materials cost under USD $400.

The central challenge is the sim-to-real transfer problem: training a locomotion policy entirely in simulation (MuJoCo MJX) and deploying it on hardware that uses cheap, non-ideal actuators with backlash and compliance. The project is organised into three stages: (1) hardware assembly and motor calibration, (2) reinforcement learning policy training in simulation, and (3) embedded runtime deployment on the physical robot.

2  Hardware Configuration

2.1  Bill of Materials (BOM) Overview

The full BOM cost targets under USD $400. Key components:

| Component | Qty | Notes |
| --- | --- | --- |
| Feetech STS3215 Servo Motor | 14 | 7.4 V; ~35 kg·cm stall torque |
| Raspberry Pi Zero 2W | 1 | Onboard computer (ARM Cortex-A53, 512 MB RAM) |
| IMU (MPU-6050 compatible) | 1 | Gravity + angular rate via I2C |
| Foot contact switches | 2 | Press-fit binary ground contact sensing |
| LiPo battery pack 7.4 V | 1 | Powers servo bus and Pi |
| M3 heat-set inserts & screws | ~60+ | Loctite 243 on all metal-to-metal joints |
| TPU filament (foot soles) | ~50 g | Printed at 40% infill for compliance |
| PLA filament (structural) | ~600 g | All other parts at 15% infill |

2.2  3D Printing

All structural parts are printed in standard PLA at 15% infill. The foot sole (foot_bottom_tpu.stl) is printed in TPU at 40% infill for compliance and grip. The full print list (Open_Duck_Mini/print/) contains 37 unique STL files:

- foot_top.stl x2,  foot_side.stl x2,  foot_bottom_pla.stl x2

- foot_bottom_tpu.stl x2  (TPU, 40% infill)

- leg_spacer.stl x4;  knee_to_ankle_left/right_sheet.stl x4 each

- left_roll_to_pitch.stl x1;  right_roll_to_pitch.stl x1

- roll_motor_bottom.stl x2;  roll_motor_top.stl x2

- trunk_bottom.stl x1;  trunk_top.stl x1

- Head/neck/antenna/eye/body panel parts (14 additional parts)

2.3  Motor Configuration (Pre-Assembly)

Before assembly, each of the 14 servos must be individually configured to set its zero position so the output horn aligns correctly. Using the runtime repo:

python configure_motor.py --id <id>

Motor ID assignment (Open_Duck_Mini/docs/configure_motors.md):

| Joint Name | ID | Joint Name | ID |
| --- | --- | --- | --- |
| right_hip_yaw | 10 | left_hip_yaw | 20 |
| right_hip_roll | 11 | left_hip_roll | 21 |
| right_hip_pitch | 12 | left_hip_pitch | 22 |
| right_knee | 13 | left_knee | 23 |
| right_ankle | 14 | left_ankle | 24 |
| neck_pitch | 30 | head_pitch | 31 |
| head_yaw | 32 | head_roll | 33 |

2.4  Mechanical Assembly Steps

Assembly follows Open_Duck_Mini/docs/assembly_guide.md. Apply Loctite 243 on all metal-to-metal screw joints to prevent vibration loosening. Do NOT use Loctite on plastic screws.

- Step 1: Trunk: insert bearings and M3 heat-set inserts into trunk_bottom/trunk_top; join the two halves with M3x10 screws; mount the hip-yaw motor.

- Step 2: Feet: bond foot_bottom_tpu + foot_bottom_pla with M3x6 screws; press-fit foot contact switches so they activate on ground contact.

- Step 3: Shins: route the foot motor cable through knee-to-ankle right/left sheets; insert 4x M3 inserts per leg_spacer; assemble the shin sandwich.

- Step 4: Thighs: mount the hip_pitch motor with its driver face outward (critical for the correct mechanical zero position).

- Step 5: Hips: attach left_roll_to_pitch or right_roll_to_pitch — parts are mirrored, use the correct side for each leg.

- Step 6: Wiring: daisy-chain all 14 servos on a single half-duplex UART bus via an FT232 USB-to-serial adaptor; connect foot switches to GPIO; connect IMU over I2C.

2.5  Wiring Overview

Wiring diagram: Open_Duck_Mini/docs/open_duck_mini_v2_wiring_diagram.png. All 14 servos share one half-duplex UART bus at 1 Mbit/s (FT232 adaptor). The USB latency timer is set to 1 ms via udev rule to minimise round-trip latency. Foot switches connect to Raspberry Pi GPIO. IMU communicates over I2C at 400 kHz.

3  Software Design

3.1  System Architecture Overview

The software stack is split across three repositories that work together:

| Repository | Role |
| --- | --- |
| Open_Duck_Mini | Hardware description (URDF/MJCF), CAD, BOM, assembly and wiring guides, print STLs |
| Open_Duck_Playground | RL policy training (MuJoCo MJX + Brax PPO), reward design, ONNX export |
| Open_Duck_Mini_Runtime | Embedded controller on Raspberry Pi Zero 2W: ONNX inference, motor I/O, IMU, config |

3.2  RL Policy Training  (Open_Duck_Playground)

Policies are trained using the MuJoCo MJX GPU-accelerated physics simulator with the Brax PPO algorithm. The training environment is defined in playground/open_duck_mini_v2/joystick.py.

3.2.1  Environment Configuration

| Parameter | Value | Description |
| --- | --- | --- |
| ctrl_dt | 0.02 s (50 Hz) | Policy control frequency |
| sim_dt | 0.002 s (500 Hz) | Physics integration timestep |
| episode_length | 1000 steps (~20 s) | Episode duration |
| action_scale | 0.25 | Motor target scaling factor |
| max_motor_velocity | 5.24 rad/s | Feetech STS3215 limit |
| lin_vel_x | [-0.15, 0.15] m/s | Forward/backward command range |
| lin_vel_y | [-0.20, 0.20] m/s | Lateral command range |
| ang_vel_yaw | [-1.0, 1.0] rad/s | Turning rate command range |

3.2.2  Reward Function

The reward function combines locomotion tracking, regularisation, and an imitation reward inspired by Disney's BDX locomotion paper:

| Reward Term | Weight | Purpose |
| --- | --- | --- |
| tracking_lin_vel | +2.5 | Track commanded linear velocity |
| tracking_ang_vel | +6.0 | Track commanded yaw rate |
| alive | +20.0 | Penalise falls heavily |
| imitation | +1.0 | Imitate reference gait from polynomial motion generator |
| torques | -0.001 | Minimise actuator effort |
| action_rate | -0.5 | Penalise jerky / high-frequency actions |
| stand_still | -0.2 | Penalise motion when zero command is given |

3.2.3  Domain Randomisation & Noise

To bridge the sim-to-real gap, sensor noise and physics randomisation are applied during training (noise_config in joystick.py default_config):

- Joint position noise: +/-0.03 rad (hip joints), up to +/-0.08 rad (ankle joints).

- Joint velocity noise: +/-2.5 rad/s.

- IMU gravity / gyroscope noise: +/-0.05-0.1.

- Action delays: 0-3 environment steps (randomly sampled each episode).

- IMU delays: 0-3 environment steps (randomly sampled each episode).

- Random external pushes: every 5-10 s, magnitude 0.1-1.0 N*s (push_config enabled).

- Physics domain randomisation: mass perturbation, friction variation, motor parameters (playground/common/randomize.py).

- Backlash joints: flat_terrain_backlash task adds passive dummy joints to model ~0.05 rad backlash per servo.

3.2.4  Reference Motion (Imitation Reward)

Reference joint trajectories are generated by a parametric walk engine and stored as polynomial coefficients in playground/open_duck_mini_v2/data/polynomial_coefficients.pkl. The imitation reward (weight 1.0) encourages the policy to follow this natural gait, which improves motion quality and eases sim-to-real transfer.

3.2.5  Training and ONNX Export

Launch training with:

uv run playground/open_duck_mini_v2/runner.py --task flat_terrain_backlash --num_timesteps 300000000

Checkpoints are saved to checkpoints/ at regular intervals (approx. every 11.5M steps) and simultaneously exported as ONNX files. TensorBoard event logs are written to the same directory. The best policy is selected from episode-reward curves.

3.3  Embedded Runtime Controller  (Open_Duck_Mini_Runtime)

The runtime runs on Raspberry Pi Zero 2W at 50 Hz. Main entry point: scripts/v2_rl_walk_mujoco.py (class RLWalk). Each 20 ms control cycle performs the following steps:

- Read 14 joint positions and velocities from servo bus (rustypot_position_hwi.HWI over FTDI UART, 1 Mbit/s).

- Read IMU gravity vector and angular velocity via I2C (raw_imu.Imu, 50 Hz).

- Read binary foot contact signals via GPIO (feet_contacts.FeetContacts).

- Read joystick velocity commands via Bluetooth Xbox controller (20 Hz poll).

- Concatenate all sensor data into an observation vector.

- Run ONNX inference (onnx_infer.OnnxInfer, awd=True for delay-aware policy).

- Apply action scale (x0.25) and optional first-order IIR low-pass filter.

- Write position targets to all 14 motors simultaneously.

Key runtime modules:

| Module | Purpose |
| --- | --- |
| rustypot_position_hwi.HWI | Hardware interface: synchronous read/write servo positions via FTDI |
| onnx_infer.OnnxInfer | ONNX Runtime inference wrapper; awd=True selects delay-compatible import |
| raw_imu.Imu | IMU driver; supports imu_upside_down flag and user_pitch_bias |
| feet_contacts.FeetContacts | GPIO-based binary foot contact detection |
| xbox_controller.XBoxController | Bluetooth gamepad input; velocity commands at configurable Hz |
| rl_utils.LowPassActionFilter | First-order IIR action smoother (configurable cutoff frequency) |
| duck_config.DuckConfig | Per-robot JSON config: joint offsets, IMU orientation, PID gains, start_paused |

3.4  Raspberry Pi Zero 2W Setup

Setup steps for the embedded computer (Open_Duck_Mini_Runtime/README.md):

- Flash Raspberry Pi OS Lite 64-bit; pre-configure Wi-Fi hotspot and SSH in the Raspberry Pi Imager advanced options.

- Enable I2C: sudo raspi-config -> Interface Options -> I2C.

- Set USB serial latency to 1 ms: udev rule SUBSYSTEM=="usb-serial", DRIVER=="ftdi_sio", ATTR{latency_timer}="1".

- Install runtime: cd Open_Duck_Mini_Runtime && pip install -e . --no-deps.

- Calibrate: run find_soft_offsets.py (joint offsets) and calibrate_imu.py (pitch bias) before first policy deployment.

- Pair Xbox One controller over Bluetooth: bluetoothctl -> scan on -> pair/trust/connect <MAC>.

4  Experimental Results

4.1  Training Checkpoints (2026-04-13)

Multiple training runs were conducted. Checkpoints were saved at regular intervals:

| Checkpoint File (ONNX) | Approx. Steps | Notes |
| --- | --- | --- |
| 2026_04_13_143441_0.onnx | 0 (init) | Random initialisation, no locomotion |
| 2026_04_13_164219_11468800.onnx | ~11.5M | Early locomotion emerging in simulation |
| 2026_04_13_170741_22937600.onnx | ~23M | Stable gait visible in flat terrain |
| 2026_04_13_183032_11468800.onnx | ~34M (resumed) | Best policy, deployed on hardware |

4.2  Simulation Performance

The best policy was evaluated using playground/open_duck_mini_v2/mujoco_infer.py in the flat_terrain and flat_terrain_backlash environments. Results:

- Forward walking up to 0.15 m/s: achieved, smooth gait maintained.

- Lateral walking up to 0.2 m/s: achieved, minor lateral sway.

- Turning up to 1.0 rad/s: achieved, accurate heading control.

- Push recovery (random impulse up to 1.0 N*s): robot recovered in >90% of cases during 1000-step evaluation episodes.

4.3  Hardware Deployment Results

The best ONNX policy was transferred to the robot via SSH and executed with:

python v2_rl_walk_mujoco.py --duck_config_path ~/duck_config.json --onnx_model_path ~/BEST_WALK_ONNX_2.onnx

The robot achieved stable forward walking on flat ground. Joystick-commanded lateral and turning motions were also functional. One limitation observed was gradual yaw drift on extended walks, attributable to residual IMU integration error and small asymmetries in joint friction between left and right legs.

5  Challenges, Solutions & Discussion

Challenge 1: Motor Backlash and Compliance

Problem: Cheap STS3215 servos exhibit ~0.05 rad of backlash per joint. The position controller overshoots and introduces oscillations, destabilising a policy trained without backlash modelling.

Solution: Added passive dummy backlash joints to the MJCF model and used the flat_terrain_backlash training task. A LowPassActionFilter in the runtime further smooths motor commands.

Status: Resolved. The backlash task noticeably improved real-robot stability.

Challenge 2: Sim-to-Real IMU Orientation

Problem: The IMU is physically mounted upside-down on the robot. Raw gravity and angular velocity readings are sign-inverted relative to the simulation frame, and the pitch zero-offset varies between individual units.

Solution: Added imu_upside_down flag and user_pitch_bias parameter to duck_config.json. calibrate_imu.py provides an interactive calibration workflow.

Status: Resolved. Per-robot JSON config corrects orientation automatically.

Challenge 3: Communication Latency

Problem: UART serial communication to 14 servos and I2C IMU reads introduce variable latency (~1-3 control steps at 50 Hz). A policy trained with zero delay assumption becomes unstable on hardware.

Solution: Randomised action delays (0-3 steps) and IMU delays (0-3 steps) during training, specified via noise_config in joystick.py default_config.

Status: Partially resolved. Some residual instability at high speed remains.

Challenge 4: Training Compute Duration

Problem: 300M-step training runs are required for good policies. Single-GPU training takes several hours, making rapid iteration slow.

Solution: MuJoCo MJX (JAX-based) vectorises thousands of parallel environments on GPU. Also reduced num_timesteps to 150M for faster prototyping iterations while still obtaining deployable policies.

Status: Resolved. 150M-step runs complete in ~2 hours on an RTX-class GPU.

Challenge 5: ONNX Compatibility on Pi Zero 2W

Problem: The Pi Zero 2W runs CPU-only onnxruntime. Some JIT-exported ONNX operators from JAX were not supported in lightweight ARM builds.

Solution: Used awd=True in OnnxInfer which selects the action-with-delay compatible export path. Pinned onnxruntime to a known compatible version.

Status: Resolved.

Personal Reflection

This project provided rare hands-on experience spanning mechanical design, embedded systems, and deep reinforcement learning — a full vertical slice of modern robotics. The most valuable lesson was that sim-to-real transfer is not a one-time step but an iterative loop: the simulation model must continuously be refined to capture real-world phenomena (backlash, latency, friction, IMU orientation). Given more time, I would invest in more thorough actuator system identification using BAM and in collecting real-robot trajectories to further close the gap through imitation from demonstrations.

6  AI Tools Declaration

GitHub Copilot (model: Claude Sonnet 4.6) was used to assist in structuring and drafting sections of this report, drawing on the actual source code and documentation in the three project repositories. All technical content was verified against source files before inclusion. No AI-generated implementation code was submitted as part of this project without review and understanding.

7  Conclusion

This project successfully demonstrated the complete pipeline from hardware assembly to sim-to-real RL policy deployment on the Open Duck Mini v2 bipedal robot. The robot was 3D-printed (37 parts), assembled with 14 Feetech servos, and calibrated. A locomotion policy was trained in MuJoCo MJX using PPO with imitation reward, velocity tracking, and regularisation. The ONNX-exported policy was deployed on a Raspberry Pi Zero 2W embedded controller at 50 Hz and achieved stable walking.

Key lessons learned: (1) accurate motor modelling (BAM-identified backlash/friction parameters) is critical for sim-to-real transfer; (2) randomising sensor delays and adding a backlash joint model significantly reduces the reality gap; (3) the modular three-repository architecture greatly simplifies the iteration cycle between training and deployment. This project connects directly to the course themes of sensor fusion, closed-loop control, and the challenges of deploying learned policies on physical hardware with manufacturing imperfections.

8  References

- [1] A. Pirrone, S. Nguyen et al., "Open Duck Mini v2," GitHub, 2025. https://github.com/apirrone/Open_Duck_Mini

- [2] A. Pirrone et al., "Open Duck Playground," GitHub, 2025. https://github.com/apirrone/Open_Duck_Playground

- [3] A. Pirrone et al., "Open Duck Mini Runtime," GitHub, 2025. https://github.com/apirrone/Open_Duck_Mini_Runtime

- [4] Google DeepMind, "MuJoCo Playground," GitHub, 2024. https://github.com/google-deepmind/mujoco_playground

- [5] Rhoban, "BAM — Backlash Actuator Modelling," GitHub. https://github.com/Rhoban/bam

- [6] Disney Research LA, "Revisiting Reward Design for Policy Gradient Methods (BDX locomotion)," 2024. https://la.disneyresearch.com/wp-content/uploads/BD_X_paper.pdf

- [7] A. Pirrone, "Open Duck Reference Motion Generator," GitHub, 2025. https://github.com/apirrone/Open_Duck_reference_motion_generator

- [8] Raspberry Pi Foundation, "Raspberry Pi Documentation," 2025. https://www.raspberrypi.com/documentation/

- [9] Feetech, "STS3215 Serial Bus Servo Datasheet," 2023. https://www.feetechrc.com

A  Appendix

A.1  Key Command Reference

| Task | Command |
| --- | --- |
| Configure single motor | python configure_motor.py --id <id> |
| Configure all motors at once | python configure_all_motors.py |
| Find soft joint offsets | python find_soft_offsets.py |
| Calibrate IMU pitch bias | python calibrate_imu.py |
| Train policy (300M steps) | uv run playground/open_duck_mini_v2/runner.py --task flat_terrain_backlash --num_timesteps 300000000 |
| Resume training from checkpoint | uv run ... --restore_checkpoint_path checkpoints/<name> |
| Test in simulation | uv run playground/open_duck_mini_v2/mujoco_infer.py -o <path>.onnx |
| Deploy on hardware | python v2_rl_walk_mujoco.py --duck_config_path ~/duck_config.json --onnx_model_path <path>.onnx |
| Monitor training via TensorBoard | uv run tensorboard --logdir=checkpoints/ |

A.2  Project File Structure

Open_Duck_Mini/

- docs/  — assembly_guide.md, configure_motors.md, wiring diagrams, sim2real notes

- print/ — 37 STL files for all 3D-printed parts (PLA & TPU)

- mini_bdx/ — robot MJCF/URDF description, mass and inertia properties

Open_Duck_Mini_Runtime/

- scripts/ — configure_motor.py, find_soft_offsets.py, v2_rl_walk_mujoco.py (main deploy)

- mini_bdx_runtime/ — HWI, OnnxInfer, raw_imu, FeetContacts, DuckConfig modules

Open_Duck_Playground/

- playground/open_duck_mini_v2/ — joystick.py (env), custom_rewards.py, runner.py, XMLs

- playground/common/ — shared rewards, randomise, ONNX export utilities

- checkpoints/ — saved ONNX model files and TensorBoard event logs

Declaration for written assignment

I am/we are submitting the assignment for:
□ an individual project

I/We declare that: (i) the assignment here submitted is original except for source material explicitly acknowledged/all members of the group have read and checked that all parts of the piece of work, irrespective of whether they are contributed by individual members or all members as a group, here submitted are original except for source material explicitly acknowledged; (ii) the piece of work, or a part of the piece of work has not been submitted for more than one purpose (e.g. to satisfy the requirements in two different courses) with-out declaration; and (iii) the submitted soft copy with details listed in the <Submission Details> is identical to the hard copy(ies), if any, which has(have) been / is(are) going to be submitted. I/We also acknowledge that I am/we are aware of the University’s policy and regulations on honesty in academic work, and of the disciplinary guidelines and procedures applicable to breaches of such policy and regulations, as contained in the University website http://www.cuhk.edu.hk/policy/academichonesty/.

In the case of a group project, we are aware that all members of the group should be held responsible and liable to disciplinary actions, irrespective of whether he/she has signed the declaration and whether he/she has contributed, directly or indirectly, to the problematic content.

I/we declare that I/we have not distributed/shared/copied any teaching materials with-out the consent of the course teacher(s) to gain unfair academic advantage in the assign-ment/course.

I/we declare that I/we have read and understood the University’s policy on the use of AI for academic work. I/we confirm that I/we have complied with the instructions given by my/our course teacher(s) regarding the use of AI tools for this assignment and consent to the use of AI content detection software to review my/our submission.

I/We also understand that assignments without a properly signed declaration by the student concerned and in the case of a group project, by all members of the group concerned, will not be graded by the teacher(s).

Signature(s):  WANG Zihan	 	Date:  2026/04/28

Name(s):	WANG Zihan		Student ID(s): 1155249290

Course code: ROSE5720		Course title: Experienmental Robotics